Projet PredCOMPACT

Démarche 1 : Analyse de l’étude du marché – choix des matières-premières

Répondre le WP1 projet ANR  -Definition of “model” compact powder formulations

Idée de cette partie est d’utiliser les données récupérées par l’analyse du marché réalisé sur la plateforme Cosmetikwatch  voir fichier Excel « compacts 170325 » qui contient les informations ci-dessous.

Liste des informations collectées à partir de l’analyse du marché sur la Plateforme Cosmetikwatch.

- Nom produit

- Marque

- Groupe(s) / Société(s) cosmétique(s)

- Gamme

- Circuit(s) de distribution

- Cible(s) consommateur

- Date de lancement

- Catégorie(s) cosmétique(s)

- Zone(s) d'application

- Type(s) de produit - Formulation(s) / Galénique(s)

- Cible(s) cosmétique(s)

- Revendications principales / claims

- Ne contenant pas / Sans

- Argumentaires de vente

- Tolérance(s) du produit

- Tests d'objectivation

- Conseils d'utilisation

- Ingrédients

- Article(s) de conditionnement / Packaging

- Contenance

- Prix moyen de vente

- PAO

- Labels / Prix Beauté

- Made in

- Note YUKA/100

- Ingrédients d'origine naturelle (%)

- Code EAN

Les besoins sont :

- Dans un premier temps, étudier le lien entre le type de produit, les ingrédients et les revendications

- Dans un deuxième temps (le besoin principal), savoir quels sont les ingrédients les plus retrouvés dans les formulations qui sont sur le marché, évaluer le lien avec les revendications. Cela permettra de nous aider à sélectionner les ingrédients pour l’étude de l’impact de la formulation sur les propriétés des compacts.

A savoir :

- La composition des poudres compacts est basée sur un mélange des poudres + liants + pigments + actifs.

- Après traitement du fichier Excel, nous devons classer les ingrédients sur les différents classes (poudres + liants + pigments + actifs)

- Puis, nous devons classer les poudres selon leurs origines. Les liants selon leurs origines et polarités.

- Les différents types des matières-premières choisis à partir de ce traitement seront commandés sur différents fournisseurs et caractérisés : poudres (propriétés physico-chimiques : granulométrie, morphologie, surface spécifique, porosité, état cristallin…) et liants (viscosité, mouillabilité et analyses sensorielles)  les résultats de toutes les caractérisations seront regroupés dans un fichier Excel

- Après les caractérisations, une sélection sera réalisée pour préparer les compacts (voir plan d’essai tableau ci-dessous).

- Les poudres compactes obtenues seront caractérisées (aspect visuel, dureté, résistance à la rupture et propriétés sensoriels)  les résultats de toutes les caractérisations seront regroupés dans un fichier Excel

Objectif : étudier le lien entre les propriétés des matières-premières et les caractéristiques des compacts  traitement de tous les résultats avec IA

| Types of ingredients and concentration range |  | Experimental factors | Suppliers number |
| --- | --- | --- | --- |
| Powders (qsp 100%) | Minerals | 3 | 10 |
|  | Vegetables | 3 | 5 |
|  | Synthetic | 10 | 3 |
| Blinders (2-4%) | Fatty alcohols | 3 | 5 |
|  | Fatty ester | 10 | 5 |
|  | Silicone | 3 | 5 |
|  | triglycerides | 3 | 5 |
|  | Mineral oil | 1 | 5 |
|  | Vegetable oil | 10 | 3 |
| Pigments (2-5%) | Iron red | 5 | 2 |
|  | Iron yellow | 5 | 2 |
|  | Iron black | 5 | 2 |
|  | Titanium dioxide | 5 | 2 |

Exemple de l’idée de matrice à construire pour le plan d’essai* :

* Créer la « database »  Such a database will consist of n=200 samples described by a large number p (p=25) of descriptors (physicochemical properties of raw materials; physicochemical, mechanical and sensory characteristics of compact powders and compaction profile data) representing an average of 5000 data.

Voici la partie WP1 du projet ANR

WP1 – Characterization of raw materials and production of model compacts – duration: 30 months.

Objectives:

- Definition of “model” compact powder formulations

- Physicochemical characterization of raw materials

- Development of compaction process – study of operating conditions

- Application of classical methods for characterization of compacts

Task 1.1 Choice of raw materials and conception of “model” formulations

The first step of this study will be dedicated to the choice of raw materials i.e. powders and binders to be included in the compact formulations. As described in part I.b, the composition of powders revolves around three families of powders: mineral (talc, mica, kaolin), vegetable (zea mays starch, tapioca starch, cellulose…) and synthetic origin (silica, nylon, lauroyl lysine, boron nitrite, alumina, magnesium myristate…). These powders will also be chosen by their function: filler powder (talc, mica, kaolin, synthetic fluorphlogopite,…), synthetic powder for touch (nylon, boron nitride), lubricant (silica), soft touch agent (lauroyl lysine), lubricant and slippery touch (magnesium stearate). Considering that the properties of these ingredients such as their particle size, specific surface area, morphology, etc., as well as the existence or not of a surface treatment can impact the compact properties, these criteria will be included in the study. Therefore, for each type of powder, we will select samples from different suppliers with varying characteristics to study their impact on the final compact properties. This approach was carried out in our preliminary study concerning the influence of talc. In this former study (CARATS/Carnot project managed by scientific coordinator), 23 talc references were obtained from 6 suppliers. The binders will be chosen according to their polarity and capacity to wet the powder mixture to ensure good cohesion between particles. Fatty alcohols (octyldodecanol,...), fatty esters of different families (isopropyl myristate, octyldodecyl Lactate, glyceryl caprylate/caprate, octyldodecyl myristate, isononyl isononate,...), silicones (dimethicone, phenyl trimethicone …) triglycerides (caprylic/capric triglycerides…), mineral (paraffinum liquidum) and vegetable oils (cocos nucifera oil/coconut oil, ricinus communis (castor) seed oil) will be selected. The type of pigment and their surface treatment will also be studied. In the composition of a powder foundation, the iron oxides (red, yellow, and black) are the most used pigments to give the color of the product, and titanium dioxide as a mattifier.

For a given compaction process, the formulation parameters to be studied include various types of powders, binders, and pigments in terms of their physicochemical properties and concentrations. Based on the literature [12,14,18] and on an analysis of the market (described in part I.b), we have identified 67 different raw materials (experimental factors) that can be used in combination in the formulation of compact powders (Table 3). Considering that i) each raw material may be sourced from different suppliers and therefore have different physicochemical properties and ii) at least 2 concentrations must be tested for each factor (by example, the lower and upper limits of their concentration range), all possible combinations of these factors can result in more than 2 million possible formulations. A representative subset of 200 formulations will be selected from this large set of candidates by using machine learning methods. These representative formulations will be prepared (Task 1.3) and characterized (Task 1.4, WP2 and WP3). The whole set of experimental data will be used to build the database to be analyzed by machine learning methods. Such a database will consist of n=200 samples described by a large number p (p=25) of descriptors (physicochemical properties of raw materials; physicochemical, mechanical and sensory characteristics of compact powders and compaction profile data) representing an average of 5000 data. Addressing such "small n, large p" problem in data science requires employing various machine learning models and techniques designed to handle high-dimensionality and mitigate the risk of overfitting. Regularized regression models, support vector machines, ensemble methods, and Bayesian approaches are integral in managing large feature sets derived from limited samples. Concurrently, principal component analysis is a cornerstone for dimensionality reduction, facilitating more effective model training and interpretation. Additionally, WP4 encompasses creating synthetic data from initial samples. This synthetic dataset amplifies the scope for AI model experimentation, paving the way for the establishment of a refined and accessible database primed for community use and collaborative advancement.

Démarche 2 : Traitement des résultats du projet CARATS

Pour cette partie, je ne sais pas si nous pourrons faire plus de ce que nous avons déjà fait.

Il s’agit de l’étude sur l’impact du type de talc sur les propriétés des compacts. Dans cette étude, nous avons

Les données de cette étude sont regroupées dans le fichier Excel « Compacts analyse de donnés_060721_donnés écoulement » (ce fichier est un exemple de comment nous allons regrouper nos résultats sur les matières-premières et les compacts). Dans ce fichier, il y a les résultats de l’analyse sensorielle des compacts, puis propriétés physico-chimiques des talcs et des compacts.

Par analyse en PCA, nous avons obtenu les résultats ci-dessous.

Présentés dans le projet ANR :

Our research team has studied the impact of the physicochemical properties of talcs on the characteristics of compacts (preliminary results by CARATS/Carnot). In this study, 23 talcs were characterized considering of color, size distribution, morphology, density, specific surface, crystalline state and flowability. The results indicate that the colour parameters can be influenced by the average diameter of talc particles, the smaller the particle size the higher the clarity. This was also correlated with specific surface area. Sensory evaluation of the compacts was done by panelists (10 persons) according to their visual appearance, smoothness on application, slipperiness, covering, spreading and smoothness after the application. Principal component analysis (PCA) of the sensory data of compacts classified them in three groups (Fig. 2). Each group sensory data is correlated to the physicochemical properties of the talcs, mainly the particle size.

| (a) | (b) |
| --- | --- |

Fig. 2. Loadings (a) and score (b) representation calculated from PCA of the sensory data of the compacts (1 control (D1) and 22 test samples (named A-F)) prepared with 23 different talcs. The colors green, orange and black represent the three groups of the compacts (preliminary results by CARATS/Carnot, not published).